INSTALLATION FOR SELF BALLASTED GLOBE

INSTALLATION FOR REPLACEMENT GLOBE & BALLAST

| Step1 To remove ballast from globe hold top of ballast and top of lamp fitting and twist anti clockwise (do not hold or twist globe). | Step 2 Now that ballast is discounted separate from globe. | Step 3 To re install ballast to globe, line up locking devices so that the contacts on the lamp holder end are at 90 degrees to the contacts on the lamp end. Twist clock-wise to lock in position. |
| --- | --- | --- |
|  | LOCKING DEVICE Never handle globe when fixing ballast to globe. This will cause breakage |  |

Before installing new globe or ballast, turn power off and disconnect self ballasted lamp from lamp holder.